Data availability for manuscript submission to Peer J.

Files Included:

Data from Dephy ExoBoots using a custom proportional myoelectric controller: EMG_EXO_DATA.zip

Map from Google which shows one lap of the outdoor course: terrain.jpg

Subject sex, height, weight, age, and walking speeds: Subject_Information.xlsx

Experiment Information:

Powered conditions were performed with a custom proportional myoelectric controller using the soleus EMG as an input and implemented via a Raspberry Pi 4.0.

Controller information will soon be available on Github via rhybart.

Conditions are as follows:

Exopowin: 6 minutes of walking on a treadmill with the Dephy ExoBoots powered.

ExoUnpowin: 6 minutes of walking on a treadmill with the Dephy ExoBoots donned, but unpowered.

NoExoin: 6 minutes of walking on a treadmill with only the boots on.

Stand: 6 minutes of standing to get baseline metabolic cost.

Exopowout: 3 laps of a mostly flat outdoor course, totaling 1.35 km with the Dephy ExoBoots powered.

NoExoout: 3 laps of a mostly flat outdoor course, totaling 1.35 km with only the boots on.

Excel file information:

Headers names in each excel file and a description of the data:

LTime: The time reported from the raspberry pi at each loop of the control loop associated with the left exoskeleton

LExoTime:  The time reported from the exoskeleton at each loop of the control loop associated with the left exoskeleton

LSORaw: Raw soleus EMG from the left leg

LTARaw: Raw tibialis anterior EMG from the left leg

LSOFilt: Soleus data that was highpass filtered at 50 Hz using a 2nd order butterworth filter, full wave rectified and low pass filtered at 8 Hz using a 2nd order butterworth filter.

LSetCurrent (mA): The desired current determined by multiplying the LSOFilt by a subject specific gain which was then sent to the left exoskeleton as a control signal.

LMotorCurrent (mA): The current reported back from the exoskeleton as what was commanded.

LMotorVoltage (mV): The voltage of the exoskeleton motor.

LAnkleAngle (deg*100): The ankle angle reported from the exoskeleton.

LMotorAngle (motor ticks): The motor angle from the exoskeleton.

LLG: Raw lateral gastrocnemius EMG from the left leg.

LMG: Raw medial gastrocnemius EMG from the left leg.

LAX, LAY, LAZ: accelerations in the x,y,z direction from the accelerometer on board the exoskeleton

LGX, LGY, LGX: rotations in the x,y,z direction from the gyroscope on board the exoskeleton

LFSR: left foot switch state: 1= weight on the foot, 0= no weight on the foot

RTime: The time reported from the raspberry pi at each loop of the control loop associated with the right exoskeleton

RExoTime:  The time reported from the exoskeleton at each loop of the control loop associated with the right exoskeleton

RSORaw: Raw soleus EMG from the right leg

RTARaw: Raw tibialis anterior EMG from the right leg

RSOFilt: Soleus data that was highpass filtered at 50 Hz using a 2nd order butterworth filter, full wave rectified and low pass filtered at 8 Hz using a 2nd order butterworth filter.

RSetCurrent (mA): The desired current determined by multiplying the RSOFilt by a subject specific gain which was then sent to the left exoskeleton as a control signal.

RMotorCurrent (mA): The current reported back from the exoskeleton as what was commanded.

RMotorVoltage (mV): The voltage of the exoskeleton motor.

RAnkleAngle (deg*100): The ankle angle reported from the exoskeleton.

RMotorAngle (motor ticks): The motor angle from the exoskeleton.

RLG: Raw lateral gastrocnemius EMG from the right leg.

RMG: Raw medial gastrocnemius EMG from the right leg.

RAX, RAY, RAZ: accelerations in the x,y,z direction from the accelerometer on board the exoskeleton

RGX, RGY, RGX: rotations in the x,y,z direction from the gyroscope on board the exoskeleton

RFSR: left foot switch state: 1= weight on the foot, 0= no weight on the foot

Sync: square wave produced by the raspberry pi sent to external devices for data synchronization

Event: 1= event marked by the operator in the GUI for a note worthy step, or error